# ECA5602- 5g/6g communication

Slot D

M.Pregna

192412350

Experiment 1: Simulation and Comparative Analysis of FDMA, TDMA and CDMA:  Resource Allocation, User Capacity and Mobile-Generation Context

Objective 1:

Code:

>> % Comparison of FDMA, TDMA and CDMA

% Simplified illustrative model; not a full cellular-network simulation.

clear; clc; close all;

% Illustrative system parameters

totalBandwidthHz = 1e6; % Total bandwidth: 1 MHz

fdmaChannelHz = 200e3; % FDMA channel width: 200 kHz

tdmaSlots = 8; % Number of TDMA slots per frame

processingGain = 64; % CDMA processing gain

targetSIRdB = 7; % Minimum acceptable CDMA SIR

% Capacity estimates under the stated assumptions

fdmaUsers = floor(totalBandwidthHz / fdmaChannelHz);

tdmaUsers = tdmaSlots;

targetSIR = 10^(targetSIRdB / 10);

cdmaUsers = floor(1 + processingGain / targetSIR);

fprintf('Illustrative maximum supported users\n');

fprintf('FDMA: %d users\n', fdmaUsers);

fprintf('TDMA: %d users\n', tdmaUsers);

fprintf('CDMA: %d users at a target SIR of %.1f dB\n\n', ...

cdmaUsers, targetSIRdB);

% CDMA SIR as the number of simultaneous users increases

K = 2:30;

sirLinear = processingGain ./ (K - 1);

sirDb = 10 * log10(sirLinear);

% Display a comparison table

method = ["FDMA"; "TDMA"; "CDMA"];

supportedUsers = [fdmaUsers; tdmaUsers; cdmaUsers];

results = table(method, supportedUsers);

disp(results);

% Plot estimated user capacity

figure;

bar(categorical(method), supportedUsers);

ylabel('Illustrative maximum supported simultaneous users');

title('FDMA, TDMA and CDMA: Capacity Estimate');

grid on;

% Plot estimated CDMA SIR as users are added

figure;

plot(K, sirDb, 'o-', 'LineWidth', 1.5);

hold on;

yline(targetSIRdB, '--r', 'Target SIR');

xlabel('Number of simultaneous CDMA users (K)');

ylabel('Estimated SIR (dB)');

title('Estimated CDMA SIR versus Number of Users');

grid on;

hold off;

output:

Objective 2

Objective 3:

Code:

generation = ["1G"; "2G"; "2G"; "3G"; "4G"; "5G"];

system = ["AMPS"; "GSM"; "IS-95"; "UMTS"; "LTE"; "5G NR"];

accessMethod = ["FDMA"; "TDMA"; "CDMA"; "WCDMA"; "OFDMA"; "OFDMA-based"];

keyIdea = ["Separate frequency channels"; ...

"Separate time slots"; ...

"Code-based user separation"; ...

"Wideband code-based access"; ...

"Orthogonal frequency resources"; ...

"Flexible OFDM-based radio access"];

evolutionTable = table(generation, system, accessMethod, keyIdea);

disp(evolutionTable);

output:

Objective 4

Code:

figure;

tiledlayout(1,3);

% FDMA: users occupy different frequency channels

nexttile;

hold on;

colors = lines(4);

for u = 1:4

rectangle('Position',[0, u-1, 1, 0.8], ...

'FaceColor',colors(u,:), 'EdgeColor','k');

text(0.5, u-0.6, sprintf('User %d',u), ...

'HorizontalAlignment','center');

end

xlim([0 1]); ylim([0 4]);

xlabel('Time');

ylabel('Frequency channel');

title('FDMA');

set(gca,'YTick',[]);

hold off;

% TDMA: users occupy different time slots on one frequency channel

nexttile;

hold on;

for u = 1:4

rectangle('Position',[u-1, 0, 0.8, 1], ...

'FaceColor',colors(u,:), 'EdgeColor','k');

text(u-0.6, 0.5, sprintf('User %d',u), ...

'HorizontalAlignment','center');

end

xlim([0 4]); ylim([0 1]);

xlabel('Time slot');

ylabel('Frequency channel');

title('TDMA');

set(gca,'YTick',[]);

hold off;

% CDMA: users share the same time and frequency band using different codes

nexttile;

hold on;

for u = 1:4

rectangle('Position',[0, 0, 1, 1], ...

'FaceColor',colors(u,:), 'FaceAlpha',0.22, ...

'EdgeColor',colors(u,:), 'LineWidth',1.5);

end

text(0.5, 0.62, 'Users 1-4 share', 'HorizontalAlignment','center');

text(0.5, 0.40, 'band; different codes', 'HorizontalAlignment','center');

xlim([0 1]); ylim([0 1]);

xlabel('Time');

ylabel('Frequency band');

title('CDMA');

set(gca,'YTick',[]);

hold off;

sgtitle('Conceptual Time-Frequency Resource Allocation');

output:

Conclusion:

The four MATLAB programs demonstrate how FDMA, TDMA and CDMA allocate communication resources and how the assumed parameters affect their estimated user capacity. The CDMA analysis shows that estimated SIR decreases as more users share the same frequency band. The mobile-generation table and resource-allocation diagrams connect these techniques to the development of wireless communication systems. These results are based on simplified assumptions. In particular, the model assumes ideal FDMA and TDMA separation and does not include real-world effects such as fading, power control, adjacent-channel interference or guard intervals. Therefore, the results illustrate the operating principles; they should not be treated as a universal performance ranking of the three methods.