# ECA5602- 5g/6g communication

Slot D

M.Pregna

192412350

Experiment 2: MATLAB Simulation of Cellular Coverage, Cell Splitting and Handoff in a Multi-Cell Network

code:

clear; clc; close all;

% Baseline assumptions

serviceAreaKm2 = 5;

radiiKm = [1.0 0.5];

usersPerCell = 30;

hexArea = @(R) (3*sqrt(3)/2) .* R.^2;

cellAreaKm2 = hexArea(radiiKm);

cellCount = ceil(serviceAreaKm2 ./ cellAreaKm2);

estimatedUsers = cellCount .* usersPerCell;

capacityTable = table(radiiKm(:), cellAreaKm2(:), cellCount(:), ...

repmat(usersPerCell,numel(radiiKm),1), estimatedUsers(:), ...

'VariableNames', {'Radius_km','CellArea_km2','EstimatedCells', ...

'UsersPerCell','IllustrativeUsers'});

disp(capacityTable);

% Plot a 7-cell cluster: central cell plus six neighbors

R = radiiKm(1);

centers = [0 0; 1.5*R sqrt(3)*R/2; 1.5*R -sqrt(3)*R/2; ...

0 sqrt(3)*R; 0 -sqrt(3)*R; -1.5*R sqrt(3)*R/2; ...

-1.5*R -sqrt(3)*R/2];

theta = (0:6)*pi/3; % flat-top hexagon vertices

figure; hold on; axis equal; grid on;

for k = 1:size(centers,1)

plot(centers(k,1)+R*cos(theta), centers(k,2)+R*sin(theta), 'k-');

text(centers(k,1),centers(k,2),sprintf('Cell %d',k), ...

'HorizontalAlignment','center');

end

xlabel('x (km)'); ylabel('y (km)');

title('Idealized Seven-Cell Hexagonal Cluster');

hold off;

% Mobile moves across the boundary between centers at x=0 and x=1.5R

mobileX = linspace(-0.2*R,1.7*R,300);

mobileY = zeros(size(mobileX));

d1 = hypot(mobileX, mobileY);

d2 = hypot(mobileX-1.5*R, mobileY);

servingCell = ones(size(mobileX));

servingCell(d2 < d1) = 2;

handoffIndex = find(diff(servingCell) ~= 0,1,'first') + 1;

figure; hold on; grid on;

plot(mobileX,d1,'LineWidth',1.5);

plot(mobileX,d2,'LineWidth',1.5);

if ~isempty(handoffIndex)

xline(mobileX(handoffIndex),'--r','Handoff decision');

end

xlabel('Mobile x-position (km)'); ylabel('Distance to cell center (km)');

legend('Cell 1','Cell 2','Location','best');

title('Nearest-Center Handoff Approximation');

hold off;

figure; stairs(mobileX,servingCell,'LineWidth',1.5); grid on;

xlabel('Mobile x-position (km)'); ylabel('Serving-cell ID');

title('Serving Cell Along the Mobile Path');

if ~isempty(handoffIndex)

fprintf('Geometric handoff at x = %.3f km\n',mobileX(handoffIndex));

end

output:

Conclusion

Summarize how cell radius changes the idealized cell count and assumed network capacity, and report the simulated handoff location. State that the model uses ideal hexagons, an area ratio count, fixed per-cell capacity and nearest-center handoff; it does not model RF propagation, overlap, fading, load or handoff timers.